|  | # JAMAICA CIVIL AVIATION AUTHORITY # TECHNICAL SPECIFICATION | THIS COLUMN TO BE COMPLETED BY TENDERER ### COMPLIANCE STATEMENT Tenderer must state below, against every item, Compliance or Non Compliance. Failure to complete and return this form may invalidate the bid. |
| --- | --- | --- |
| RFP# March 2023 |  |  |
| TITLE: Technical Specifications for the REPLACEMENT OF INSTRUMENT LANDING SYSTEM (ILS) WITH DME AT THE NORMAN MANLEY INTERNATIONAL AIRPORT IN KINGSTON JAMAICA |  |  |
| It is strictly prohibited for Tenderers to alter this document. Only the originator of the specification may provide amendments. |  | SUPPLIER NAME: |
| 1. OBJECTIVE The Jamaica Civil Aviation Authority (JCAA) intends to procure, on a fully integrated, turnkey basis, one ILS which consists of as a minimum: - One (1) dual hot/standby Localizer and associated equipment, materials and installation - One (1) dual hot/standby Glide Path and associated equipment, materials and installation - One (1) dual hot/standby Low Power DME and associated equipment co-located with the Localizer. - One (1) Local Technical Maintenance Display and Processing System for (LCMS) local control and monitoring system - One (1) Remote Technical Maintenance Display and Processing System (RCMS) for remote control and monitoring (Wireless connectivity solution is Preferred). - Two Near Field Monitor (NFM) system - Two Frangible shelters to house GP and LLZ Equipment - Two generators to power associated equipment with frangible shelters housing 2. SCOPE This specification covers the supply and installation of a category 1 or 2 Instrument Landing System (ILS) to serve Runway 12 at the Sangster International Airport located in Montego Bay, Jamaica. Work associated with the construction upgrades required at this facility includes but is not limited to: • Site Layout by a Licensed Land Surveyor • Locating utility sites • Site preparation and Excavation • Laying of cast in-place reinforced concrete • Concrete pull box installation • Underground electrical conduit/duct bank installation, • Electrical cable and enclosure installation, • Electrical grounding, • Antenna tower assembly and creation • Antenna array equipment assembly; Antenna and Tower demolition • Associated site work and cleanup as described herein and on the drawings. |  |  |

| SECTION A – INTENT AND STANDARDS |  |
| --- | --- |
| 3. STANDARDS - All designs, building, materials, manufacturing techniques and workmanship shall be in accordance with the highest accepted international and national standards for this type of equipment, including the construction, building codes and electrical standards for all civil works. - Where applicable, the equipment shall fully comply with, or exceed the requirements of the following documents (latest edition plus any related amendments and any other relevant documents): - ICAO Annex 10 (up to the latest Amendment) Aeronautical Telecommunications Vol. 1, III and IV; - ICAO Annex 11 International Standards and Recommendation Practices, Air Traffic Services; - ICAO Doc. 4444 Procedures for Air Navigation Services - Air Traffic Management ATM, Amendment 1; - ICAO Doc. 9426 Air Traffic Services Planning Manual; - ICAO Doc. 8071 Manual of Testing of Radio Navigation Aids; - ICAO Annex 14 Aerodromes Volume 1 Aerodrome Design and Operations. - The Tenderer shall also state, where applicable, the standards to which the whole, or any specific part, of the equipment and/or civil works complies. - The System(s) to be manufactured shall be according to the latest ISO 9001/9002 standards or equivalent standards. - The transformers, cells, distribution panels, switches, bars, wiring, fuses and all the electrical elements in general shall comply with recognized international and national standards as applicable. The Tenderer shall indicate in its proposal which standards are met by each element offered. - TENDER SPECIFICS - Alternatives 4.1.1. The Tenderer is free to offer any equipment, design or service, which in his opinion, is equal to, or superior to the requirements of this specification. Any such alternative(s) or variation(s) must be fully and clearly defined and supported so that equivalence or superiority can be readily determined. 4.1.2. All alternative(s) or variation(s) proposed shall be described and quoted separately with an explanation of the improvement which would result from their implementation. - Tender Documentation - Compliance Statement: All offers shall be accompanied by a correctly completed Compliance Statement in the form of this specification with the Tenderer indicating in the right hand column, Compliance (C) or Non Compliance (NC). If compliance is indicated, any further references, statements, comments or notes, will not waive the liability of the Tenderer on the stated Compliance. The Tenderer shall reference the compliance statement to the appropriate sections of their supporting documentation. Lack of such definitive indication for any requirement may invalidate the offer. Tender documentation is to be provided in English and in three (3) copies. The Tenderer shall provide in compact disc (CD) form an electronic copy of its tender documentation. - The Tenderer’s proposal shall be supported by adequate technical documentation including data sheets, performance sheets, drawings, illustrations so that a complete and detailed evaluation of the equipment can be made. In the proposal, the Tenderer shall present the detailed description of the equipment to be provided to the JCAA including designs, specifications and basic plans indicating all the characteristics, sources, makes, capabilities, powers, codes, standards, manufacturing processes, diagrams and other information that will show the design, quality and operating characteristics of the systems being offered. 4.2.3. Note: Whilst the attachment of brochures and supporting literature is strongly encouraged, and may in some cases be necessary to illustrate certain features of the product, it does not relieve the Tenderer of the obligation to fully complete the compliance statement of this specification as indicated above. 4.2.4. Supporting documentation shall clearly identify by a remark, any optional equipment and features that are not included in the proposal. 4.2.5. The Tenderer shall furnish to the JCAA, the necessary instructions for the assembly, operation, maintenance, and lists of spare parts of the elements and accessories that will be provided for the implementation of the system under this procurement. 4.2.6. The Tenderer shall provide the necessary documents, permitting the evaluation of the quality and functionality of the work proposed for the project infrastructure. 4.2.7 The financial offer shall provide prices in USD itemised to the Line Replaceable Unit (LRU) level. - Experience - The Tenderer, and the different companies under partnership or in a contractual agreement with the Tenderer, shall preferably be ISO 9001 certified. - The Tenderer shall demonstrate previous experience in designing, engineering and implementing systems of the same or similar type and scope of that is being proposed. - The Tenderer shall appoint properly qualified personnel in sufficient number to perform the work within the proposed timeframes. - The Tenderer shall provide the organizational chart of the company and resumes of key personnel of its technical staff. - The Tenderer shall appoint a Senior System Engineer. The Senior System Engineer shall be responsible for the design and integration of all System. - The Tenderer shall appoint a Program Manager who shall manage the implementation, and all operational aspects of the System. This Program Manager shall be the point of contact for all aspects of the System operations. - The Program Manager shall have a Project Management Certification (PMP) or global equivalent. - The Tenderer shall indicate the locations and provide full contact information including, address, telephone/fax numbers and email addresses of contacts in at least four (4) locations where the Tenderer has installed equipment and systems similar to the ones offered for this project in the last 5 years. This information must be precise as representatives from the JCAA may visit those sites to verify the operation of this equipment. - Mandatory Site Survey - For tenderers not familiar with the Norman Manley International Airport it is recommended that a site survey is conducted at the tenderer’s own expense prior to submission of a tender. The design shall take into consideration all local constraints and site particularities. Lack of knowledge of the exact local conditions will not absolve the Tenderer, under any circumstance, from fulfilling his contracted mandate. All site visits shall be coordinated through the JCAA. A synopsis of the site visit shall be included in the proposal. - The Tenderer shall coordinate and schedule the Site Survey through the appropriate JCAA personnel. |  |

| SECTION B – GENERAL EQUIPMENT CONDITIONS |  |
| --- | --- |
| 1. ENVIRONMENTAL CONDITIONS 1.1. The ILS, shall be fully protected against the effects of destructive ambient conditions and shall be designed for continuous operation during the expected life cycle (as expected by the Manufacturer) under the following environmental conditions: Operating installed in controlled environment Temperature : +10 to +40 degrees C Relative Humidity : 30 to 90% Altitude : 0 to 5,000 feet Not operating installed indoors without damage Temperature : -10 to +60 degrees C Relative Humidity : 5 to 100% Altitude : 0 to 5,000 feet Installed outdoors Temperature : -20 to +70 degrees C Relative Humidity : 5 to 100% Altitude : 0 to 5,000 feet Wind : 0 to 157 km/h operating 0 to 330 km/h non-operating Rainfall Operational : 1-hour average rain rate of 100 mm with maximum wind speed of 65 km/h Rainfall Nonoperational : 1-hour average rain rate of 130 mm with maximum wind speed of 118 km/h 1.2 Atmospheric Salt and Corrosion - The ILS systems shall be able to withstand very high atmospheric salt environment. The tenderer shall state in its proposal the material and treatment used during manufacturer and installation. - It is essential that the external parts of the system be adequately protected to prevent corrosion and degradation. Tenderers shall state in its proposal the methods, which shall be used to minimise these effects. - All bolts, nuts, washers, rivets used in the construction of the shelter, antenna and masts shall be of corrosion resistant material (e.g. stainless steel, hot dip galvanizing steel/aluminium) and impervious to rust. - Tenderers shall provide details on inspection schedules, duration and frequency. - GENERAL CONSTRUCTION - The hardware to be provided for this project shall be off-the-shelf, state-of-the-art and of proven commercial quality. To assure local commercial availability of maintenance skills and spare parts, it is required that the computers offered by the Tenderer are of a make and type in common use in Jamaica. Any equipment and accessories to be mounted shall use standard 48.26 cm (19 inches) rack mount format. - Structural strength and rigidity of equipment units and cabinets shall be such that handling for loading, shipping, unloading and setting into position for installation shall not result in any deformation sufficient to impair the appearance of any cabinets or consoles or to interfere with ease of maintenance, removal of units, ventilation, and operation of access doors. - Where equipment is mounted in cabinets, slides shall be incorporated to permit withdrawal of the units towards the front. - To facilitate their installation in the designated locations, the contractor shall take into consideration the access clearances (doors, stairs, etc.) in order to determine the maximum dimensions of the equipment in case they need to be dismantled. - A modular concept shall be used to the maximum possible extent to facilitate fault location and rapid equipment restoration. Necessary extender boards or extender systems shall be provided to facilitate testing during maintenance procedures. - Any measuring and monitoring units for the control of the equipment shall be an integral part of them. - To facilitate maintenance, all the cables shall be properly identified through a colour code and/or labeled with a method guaranteed not to deteriorate with time. - The Tender shall remove the existing equipment, shelters and foundation for the present Localizer and Glide Path to ensure it meet all required frangibility specifications. 3. ASSEMBLY AND FASTENING OF EQUIPMENT - To meet operational requirements the Tenderer shall demonstrate that the parts and sub-assemblies can be removed quickly and without any adverse effect upon system availability. - To facilitate the maintenance and replacement of parts, whenever possible, the use of the lower rack units of the technical cabinets shall be avoided. - In order to facilitate quick maintenance interventions, all panels, technical or operational bay doors (low voltage panels, dividers, control furniture, etc.) shall not have key operated locks. Magnetic Door Fasteners shall be used for this project. - All units, PCBs, assemblies and sub-assemblies should be mounted in such a way that their replacement can be installed easily and with minimum down time for the operation of the system. - All mechanical parts of the equipment assemblies shall be adequately cleaned and treated. - Material protection against overloads and over voltages caused by atmospheric phenomena shall be assured by appropriate protection devices. - All metalwork shall be sturdy. Bending radii shall be within operational standards. 4. EQUIPMENT SOFTWARE - The Tenderer shall provide a complete software package (including diagnostics and all software licenses), which is free of all known errors. Off-the-shelf proven programs and packages should be where possible. The licenses of the software must be specified with prices in the proposal. - Programs shall be designed in a structured language and based on open architecture principles. The Tenderers shall specify the language(s) and version(s) used including all operating systems (Linux shall be the operating environment used), all database management software, and program development and run time languages. - A structured or object oriented programming approach should be used to produce reliable and maintainable software which can be easily changed and expanded. All of the individual programs used to implement functions shall be modular. 5. EQUIPMENT HARDWARE - It is highly desirable that commonality of hardware be considered to simplify logistic support and reduce training costs during the life of the equipment. This especially relates to similar processors and ancillary functionality, i.e. hard disks, CD/DVDs, memory, video cards, etc. for maintenance workstations and display processors. The hardware should have a high level of availability. - Commercial Off-The-Shelf (COTS) hardware shall be used where possible. - The systems and equipment shall be capable of automatic start up at power on, a system crash and/or a power failure. It must also be possible to initiate a system start up in either a cold start or a warm start mode. - A “cold start mode” shall consist of starting the system with default parameters, with configuration and all active files cleared. - A “warm start mode” shall consist of starting the system with the latest parameters for the configuration prior to the incident and active files, except data files that have reached obsolescence. - The operative environment shall have a menu to allow the operators to modify the different parameters, such as brightness and colour - Any human machine interface shall have a graphic environment, be user friendly, and utilize windows; the information shall be displayed in different colours for easy interpretation. 6. CABLING - The Tenderer shall standardize cable types and lengths in the installations. - The cables shall be laid out, depending on the location, in cable troughs, elevated floors, suspended ceilings, pipes, or cable shafts. - Cables shall be placed side by side and tied at regular intervals along their routing, and grouped by function. - To avoid induction, low-level signal cables shall be separated from power cables by a minimum of .25 m and shall cross at perpendicular angles. - Each cable shall be identified at its extremities by a coloured label containing the following information: - Cable function - Cable number (this number shall identify the nature of the cable, its source and its destination) - The contractor shall provide the appropriate cable documentation which shall contain the following information for each cable: - Source - Destination - Cable type - Cable function - Cable number 7. QUALITY OF PARTS AND COMPONENTS - The components shall be of superior grade and must operate at lower than their nominal characteristics to increase equipment reliability. - Components shall be selected to ensure high equipment mean time between failures with respect to the site environmental conditions. - The cables shall have copper conductors and fire retardant isolation. 8. EXPANSION CAPABILITY - Reserve spare capacity of the processing time shall be not less than forty per cent (40%) of the available processing time for each processor. Based on average use values. - Reserve spare capacity of the memory shall be not less than forty per cent (40%) of the available memory for each processor. Based on average use values. - Power supplies shall have a capacity of not less than 1.5 times the capacity required for the proposed design. - Sufficient input/output communications ports shall be physically provided to meet the specified requirements plus thirty per cent (30%) expansion capability. 9. EQUIPMENT PROTECTION - All electrical, electronic, and mechanical components shall be carefully protected from impact of the most severe conditions that may prevail. - The protection of electrical and electronic equipment shall be provided particularly to withstand damage by lightning strike, humidity, high or low ambient temperatures. - The electronic equipment modules shall have the appropriate over voltage and electromagnetic interference (EMI) protection to their inputs and outputs when connected to an exterior environment. 10. SAFETY REQUIREMENTS - The equipment design and construction shall provide adequate safety for personnel and equipment during the installation, operation, maintenance and repair. All exposed voltages higher than fifty (50) volts shall be covered to avoid accidental contact on the part of the maintenance personnel. - Electromagnetic radiation levels shall meet personnel safety standards. The Tenderer will specify the standard used to meet this requirement. 11. ELECTROMAGNETIC RADIATION - The equipment will operate in its intended operational electromagnetic environment, without suffering or causing unacceptable performance degradation as a result of an electromagnetic emission or response. - The Successful Tenderer shall be responsible for conducting any required Interference and Radio Frequency transmission studies in order to obtain relevant license(s) from the JCAA Authorities. The said license(s) will be the responsibility of the Tenderer and at their own expense. 12. MAINTENANCE DESIGN - The proposed equipment shall possess hardware maintenance features to reduce repair time by providing the technical personnel with the capability to diagnose a fault rapidly and to identify the failed unit and replace it quickly in order to satisfy the availability requirements. Minimal preventive maintenance is a fundamental design requirement. - Maintenance design features shall include on-line and off-line diagnostics, power-up diagnostics, test points, Built-In Test Equipment (BITE) and Fault Isolation Testing (FIT). All equipment possessing a computer, processor or CPU shall be equipped with diagnostic programs as a part of the delivered software. - The system shall be equipped with a manual self-test to ensure comprehensive monitoring of the system performance during maintenance and troubleshooting. The self-test features shall be capable of identifying the status of the equipment up to the interchangeable module level. - The recommended maintenance concept applicable to the proposed equipment shall be indicated in the tender documentation (i.e. pre-planned, on-condition). - Wherever possible software failures shall be self-documenting, In the event a failure is detected, they system must allow for a manual crash dump to be initiated, allowing the system to be reloaded and restarted. 13. MAINTENANCE SUPPORT SERVICES - The Tenderer is requested to provide all the details and information regarding their Support Service. - The Tenderer shall describe in detail their Support Service policy and procedures in order for the client to obtain support services during and after the specified warranty period. - The Tenderer shall provide the maximum service response time (SRT) and maximum turnaround time (TAT) for each of the following services provided during and after the warranty period: - Supply of a new module or sub-assembly; - Supply of a part or a component; - Repair of a defective module; - Repair of a defective sub-assembly; - Request for technical assistance and/or support from the factory to the client site via telephone or modem line; - Request for technical assistance and/or support at the client site for support service technical personnel (engineer and/or technician); - Request for hardware and/or software modification. - The Tenderer shall provide with the equipment documentation, a “Historic Logbook” which shall be used by JCAA personnel to enter and note all interventions completed on the ILS, and all associated equipment. This Historic Logbook must record the following: - Maintenance; - Failures; - Repairs; - Modifications; and - Software troubles and updates. - The “Historic Logbook” shall remain with the equipment, for the life of the equipment, and must be in a form useful if the Tenderer wishes to retrieve vital information on failures and/or problems that occurred on the system during and after the specified warranty period. 14. REGIONAL REPRESENTATION - It is recommended that as much as possible that the equipment, computers, main components and peripherals supplied, shall have a representative regionally, - The Tenderer shall provide full contact information for its regional representative and other information including following: - Name, address, phone number, and email address; - Type of services offered(Repair and/or replacement of module or sub-assembly, maintenance services,); and - The qualifications and experience of the representative. - The Tenderer must guarantee regional representation during the life of the specified warranty. 15. INTERFACING - The Tenderer shall be responsible for the interface between the proposed equipment and all required external and existing systems including the provision and installation of any cabling and/or infrastructure. - All the data transmission/communication systems proposed by the Tenderer for this project, such as microwave links, optical fibres, shall fall within the framework of the existing regulations of acceptable international standards. 16. RELIABILITY, MAINTAINABILITY AND AVAILABILITY - The Tenderer shall, as part of the proposal, provide detailed information regarding technical specifications of each of the proposed equipment, materials, and accessories demonstrating integration among them. - The Tenderer is requested to provide the MTBF, MTTR, MTBCF and Availability Figures of all the equipment proposed and the overall system, indicating the method used for the calculation. If sub-system MTBF, MTTR, MTBCF, and Availability numbers are submitted, these shall be rolled up into an overall system MTBF, MTTR, MTBCF and Availability numbers. The Tenderer shall provide the MTBF, MTTR, and the MTBCF numbers of other sites with similar types of equipment installed so the quoted numbers can be verified by JCAA. - The Tenderer shall provide a list of parts which are unique to the system (such as the antenna and its pedestal) and which in case of failure would be considered critical and cause a suspension of service. 17. TECHNOLOGY UPGRADE - The Tenderer shall agree that the latest version of software, hardware, training materials and associated documentation shall be delivered at no extra cost if newer technology or developments occur between contract signature and time of delivery and/or installation of the equipment. This is particularly relevant to COTS hardware and software but for the system software, this applies only to software upgrades that have been included in the latest software build and has been placed under configuration management by the Tenderer. The Tenderer shall secure the JCAA’s prior written approval before any such upgrade substitution is initiated. - The Tenderer must commit to deliver the documentation corresponding to any technical upgrades carried out to the equipment, during the life cycle of the equipment. 18. GUARANTEE/WARRANTY - The Tenderer shall warrant that all equipment delivered under an eventual contract (including any equipment components manufactured by their sub-contractors) shall perform in accordance with, and conform to all specifications, descriptions, and other requirements included in their proposal, and shall be without defects in materials, workmanship, and design. If the Tenderer fails to accomplish these performance criteria, the Tenderer must modify/add and/or exchange the inadequate equipment and/or software, as necessary, to provide the specified functions. - The Tenderer’s warranty shall remain in effect for a period of two (2) years from the final commissioning date to the levels and figures achieved and approved at the time of the Provisional Site Acceptance Test and the Factory Acceptance Test. With respect to major systems and/or equipment repaired, the original period of the warranty of the said equipment shall be extended for a warranty period equivalent to the time required for the repair. - The Tenderer shall be responsible for processing all under warranty claims and will cover all expenses involved in transportation, customs clearance, shipment, repair/replacement and installation of the defective equipment to and from the Jamaica. - The warranty period shall begin anew for any equipment replaced. - The Tenderer shall submit to the JCAA a certified copy of all formal agreement(s) with local/regional appointed companies responsible for any warranty service for the equipment it will supply as a part of this project. - If any equipment fails during the warranty period, before its expiration, the JCAA shall immediately notify the Tenderer, specifying the type failure where possible. The Tenderer shall decide whether to repair the item locally/regionally or or to return it to their facilities for repair. If a spare part(s) or replacement item is not available to restore the operation of the equipment, the Tenderer, shall provide an immediate replacement of the failed item(s) in order to restore immediate operational capability at no extra cost. The Tenderer shall guarantee a thirty- (30-) day turnaround time to replace all failed item(s) of equipment (this period excludes the time for transporting the equipment and customs clearance). - If the reported problem is not solved in thirty (30) days, the JCAA shall be entitled, at its option, to: - return the equipment to the Tenderer, at the Tenderer’s own expense, and require the Tenderer to repair and re-deliver the repaired equipment to the JCAA, or - repair the equipment or have the equipment repaired by a third party and, in either case, recover the reasonable cost of repair from the Tenderer, or - Require the Tenderer to deliver replacement equipment to the JCAA. - The costs for the return and re-delivery of the equipment, as well as any repair costs, shall be at the Tenderer’s risk and expense. JCAA shall also be entitled to recover from the Tenderer all reasonable and demonstrable costs incurred in removing the repaired or replacement equipment or in assembling repaired or replacement equipment and in re-installing repaired or replacement equipment. It is understood that such warranty repair work will be carried out in-factory, or locally at the appointed representative’s facilities previously approved by the JCAA, except in the case of a system failure which might require the dispatch of Tenderer’s engineers to Jamaica or resolution through a remote maintenance connection. The import and export documentation required for transported materials shall be prepared by JCAA according to the existing national customs regulations. - If it is determined that the equipment sent to the Tenderer for repair or replacement was not defective, the transportation cost from the site to the Tenderer’s factory or local authorised representative, and return costs shall be covered by the JCAA. - This warranty does not apply for normal wear and tear items. It shall not cover equipment or parts of equipment modified after its delivery without the Tenderer’s prior written agreement. The warranty shall only apply as long as the equipment has been used and maintained in compliance with the Tenderer’s operation and maintenance manuals and under normal operating conditions. - The Tenderer shall list the item that it proposed to have normal wear and tear and is not cover under warranty. - Within the warranty period, the Tenderer confirms an unlimited number of on-site visits and maintenance assistance, on an on-call basis for warranty claims at no cost to JCAA, should the problem solution fail via remote facilities. - During the warranty period, the Tenderer shall provide on-site technical intervention and support by immediate use of local representative personnel first for all critical failures, and if necessary, shall send a specialist from its facilities within seventy-two (72) hours of the time at which the JCAA’s written request or notice for such intervention or support was sent. - The use of a local representative by the Tenderer shall not diminish the Tenderer’s responsibilities and obligations under an eventual contract. Any upgrading and modification of the equipment associated with the defaults of the equipment shall be provided by the Tenderer at no extra cost to the JCAA within the expected Life of the equipment. - If the operation or use of the materials or equipment proves to be non-compliant with the technical specifications or intended performance characteristics, the JCAA shall have the right to operate and use such materials or equipment until they can be taken out of service by the Tenderer for correction Tenderer of such defects, errors, or omissions and for replacement, in whole or in part, if correction is unsuccessful or unfeasible. - During the warranty period, if any equipment or equipment component part fails that is not included in the list of spare parts recommended by the Tenderer, the Tenderer shall supply that part or component to the JCAA as a spare part, at no additional cost to the JCAA. - The warranty period shall commence after the equipment has been received on site, site tested, commissioned and certified as accepted by the JCAA after the SAT and has been certified free from any operational or functional faults or defects. - The equipment to be supplied shall be in every aspect exactly as proposed in this tender. The performance of the equipment shall be the same as, or superior to the specifications stated in this Statement of Compliance. - The Tenderer shall quote in its proposal for an additional one (1) year warranty. - During the warranty period the Tenderer shall provide an onsite technical support personnel to assist the JCAA maintenance staff in the maintenance and servicing of the equipment. The Tenderer shall list the cost this requirement as a separate item in its proposal. 19. LIFE CYCLE SUPPORT - The Tenderer shall provide life cycle support for a minimum period of ten (10) years, starting at the expiration of the warranty period. The life cycle support shall assure that specifications and test results are maintained throughout the life cycle of the system. - The Tenderer shall provide maintenance and repair for all Line Replaceable Units (LRUs) requiring specialized repair facilities. - Provide site support, when required, to the JCAA maintenance personnel for the operation, maintenance, trouble-shooting and problem resolution for the systems. - The Tenderer shall provide documentation for the technical updating of equipment during its life cycle. - The Tenderer shall provide calibration and repair of specialized test equipment. - The Tenderer shall provide Technical, refresher and Operational training over the expected life of the equipment. - The Tenderer shall provide software upgrades over the expected life of the equipment. |  |

| SECTION C – TECHNICAL REQUIREMENTS |  |
| --- | --- |
| - Localizer The Tender shall meet or exceed the below specification for the Localizer; Specification; Characteristics. Nomenclature; Localizer Electronic Subsystem, Localizer Log Periodic Dipole Antenna, Localizer Distribution Unit and Combiner Unit.. Frequency Range; 108.1 to 111.95 MHz. Course and clearance frequency separation; 8.0 kHz (nominal). Frequency Stability; 0.0005% TYP.. Frequency Control; Direct digital synthesis (DDS) frequency synthesizer. Modulation; Amplitude modulation at 90, 150 and 1020 Hz. 90 and 150 Hz (Dual Frequency); Adjustable 0 to 25% of each tone. 1020 Hz; Adjustable 0 to 15%. Course transmitted carrier-plus-sideband (CSB) output; 15 W (Nominal) ; adjustable from 0 to 17.0 W. Course transmitted sideband-only (SBO) Output; Dependent on CSB and runway length, maximum SBO is produced only when CSB is set for maximum. Clearance transmitted CSB output; 7.5 W (Nominal); adjustable from 0 to 17.0 W. Clearance transmitted SBO output; Dependent on CSB and site conditions; maximum SBO is produced only when CSB is set for maximum. Antenna input Impedance; 50 ohms (Nominal). Antenna Input voltage standing-wave ratio (VSWR); 1.2:1. Course Width; Adjustable 2.4˚ to 7.2˚. Clearance difference in depth of modulation (DDM); DDM shall increase substantially linearly with respect to angular displacement from between 10 and 35˚ on either side of front course line where DDM is 0.180; from that angle to +/- 35˚, DDM shall not be less than 0.180. Polarization; Horizontal. Threshold displacement sensitivity; 0.00044 DDM/ft (nominal). Relative humidity; Sheltered 0 to 90%· out ide 0 to 100%. Wind loading; Up to 100 mi/h (161 km/h). Altitude; To 10,000 ft (3048 m). Duty cycle; Continuous unattended. Primary power requirements; Dual Nominal 90- 260 V AC +/- 10% 47- 63 Hz +/- 1% dual phase line-neutral with Dual Battery Backup (24Vdc). Battery requirement; 4 hrs. continuous operation (min). Battery recharge capability; Recharge 50% discharged battery within 8 hrs.. Mean time between failure; above 4000 hr (min). - Glide Slope The Tender shall meet or exceed the below specification for the Glide Slope; Specification; Characteristics. Nomenclature; Glide Slope Electronic Subsystem, Glide Slope Antenna Element, Glide Slope. Frequency range; Operates across band of329.3 to 335.0 MHz in 0.150-MHz increments. Frequency stability; ±0.002% (±0.0005% TYP.). Frequency control; Frequency synthesizer. Course transmitter modulation percentage; Adjustable 0% to 49.75% of each tone. Clearance transmitter modulation percentage; Adjustable 0% to 99.5%. Course transmitted carrier-plus-sideband (CSB) output; 3 W nominal: adjustable from 0 to 4.0 W. Course transmitted sideband-only (SBO) output; Not less than 0.15 W at maximum CSB power. Clearance transmitted CSB output; 0.5 W nominal; adjustable from 0 to 4.0 W. Antenna input impedance; 50 ohms (nominal). Antenna input voltage standing wave ratio (V SWR); Not more than 1.2:1 under normal test conditions with antenna fed from 5O-ohm line; not more than 1.3:1 over full range of service conditions. Type of radiation; Horizontally polarized amplitude-modulated radiation in a directional pattern· minimum front-to-hack ratio 16 dB; gain 10 dB above isotropic radiator. Glide angle; Adjustable 2° to 4°. Relative humidity; Sheltered, 0 to 90%; outside, 0 to100%. Altitude; To 15 000 ft (4573m). Duty cycle; Continuous unattended. Primary power requirement; Dual Nominal 90-260 V AC +/- 10% 47-63 Hz +/- 1% two-wire phase-neutral with Dual battery backup (24 Vdc). Battery requirements; 4 hrs. continuous operation (min). Battery recharge capability; Recharge 5O% discharged battery within 8 hr. Mean time between failures; above 4000 hr (min). - Distance Measuring Equipment (DME) The Tender shall meet or exceed the below specification for the DME; Specification; Characteristics. Nomenclature; Distance measuring Equipment built with a 1:1 redundancy with dual power supply and backup batteries.. RF Power Output; 100W. Primary Power (Dual Required); AC 230 V 0r 115 V +/- 15% 50 Hz DC 22-29 V. DME has in-built, or stand-alone battery charger power supply unit, Powered from mains dual BCPSU for dual system.. Power consumption (dual system); 210 W (Nom) for 100W at 1000 ppps. 300W maximum for 100W. (Maximum powers do not include the power required to charge the batteries). Frequency Range; 962 MHz to 1213 MHz. Transmit:. Receive:; 1025 MHz to 1150 MHz. X Channel; Transmit on 962 MHz to 1024 MHz. Receive on 1025 MHz to 1087 MHz. Transmit on 1151 MHz to 1213 MHz. Receive on 1088 MHz to 1150 MHz. Y Channel; Transmit on 1088 MHz to 1150 MHz. Receive on 1025 MHz to 1087 MHz. Transmit on 1025 MHz to 1087 MHz. Receive on 1088 MHz to 1150 MHz. Frequency Stability; ±0.002% (±0.0005% TYP.). Transmit:; 0.00%. Receive:; 0.00%. Spurious Output; 10 MHz to 962 MHz and 1213 MHz to 1800 MHz. Less than -40 dbm in any 1KHz bandwidth. Receiver Sensitivity; -91 dbm for 70% reply efficiency measured at the antenna cabinet connector.. Receiver threshold sensitivity; Variation < 1db for reply rate < 2400 pps. Variation < 3db for reply rate between 2400 pps and 3600 pps. Transponder Overload; Receiver AGC attenuation range > 55 db. Receiver dependent on pulse spacing; Threshold sensitivity shall not be reduced by more than 1 db for inter pulse spacing variation of +/- 1 µs. Receiver Maximum input level; + 20dbm at cabinet connector over band 1025 MHz - 1150 MHz.. Receiver sensitivity IF:; Threshold sensitivity shall not changed by more than 3 db for interrogation frequency removed 100 KHz from assigned channel.. Interrogation signals 900 KHz from channel center frequency of amplitude -10dbm to -91dbm at cabinet connector will not trigger transponder. Receiver Sensitivity RF:; IF signals at the cabinet connector are attenuated by > 80 db. Image and spurious receiver responses within the range 962 MHz - 1213 MHz are attenuated by > 75 dB. Receiver dead time; < 60 µs. Receiver Recovery time; Within 7µs of reception of test pulse threshold sensitivity < 3 db removed from value if test pulse abscent.. Decoder Rejection; The transponder shall not decode interrogation where the code deviates from nominal by > + 2µs. Pulse pair spacing transmit; 12+ 0.1 µs X channel. 36+ 0.1 µs Y channel. Reply rate; 700 to 5100 pulse pairs per second (ppps). Squitter rate; Minimum 700 ppps-. Maximum 1200 ppps. (set by PLD at 730 ppps nom). Pulse transmitter power variation; Peak power of both pulses of pulse pair differ by < 1.0 dBm.. Transmit Harmonic level; Peak power of any harmonic of the carrier frequency on any operating channel is < -20 dBm. Inter pulse Output level; RF output level shall be between constituent pulses of reply pulse pair is 80 db below level of each pulse for at least 1 µs (X mode) 15 µs (Y mode). Inter pulse pair output level; < 80 db below peak level of each pulse. Pulse spectrum (effective radiated power); < 200 mW ; 0.5 MHz band + 0.8 MHz off channel. < 2 mW ; 0.5 MHz band + 2.0 MHz off channel. Transmit Pulse shape; Pulse width at half-amplitude point 3.5 + 0.5 µs.. Pulse rise time 2.5 µs - 1µs + 0.5 µs (10% to 90%). Pulse fall time 2.5 µs -1 µs +0.5 µs (10% to 90%). Between the 95% points on the leading and trailing edges, the instantaneous amplitude does not fall between 95% of maximum amplitude.. Ident Rate; 1350 + 25 pps. Time reference; First Pulse. System reply delay; 50 µs nominal in X mode. 56 µs nominal in Y mode,. Settable over 50 + 15 µs and 56 + 7 µs in 0.05 µs increments.. System delay variation; Input signal delay Variation (at cabinet connector). -10 dbm to -81 dbm + 0.5 µs. -81 dbm to -91 dbm + 1.0 µs. The above delay variation shall be maintained for interrogation within 100kHz of nominal channel frequency, interrogation pulse rise time between 1.5 µs and 3.0 µs, reply rate varying between 700 and 5100 pps.. A multipath level of -6 db will cause a variation in transponder replay delay of less than 0.8 µs and 0.2 µs for multipath delays of <4µs and >4 µs respectively.. Battery requirements; 4 hrs. continuous operation (min). Battery recharge capability; Recharge 5O% discharged battery within 8 hr. Mean time between failures; above 4000 hr (min). - Monitoring and Control - The system shall be able to remotely monitor status of the LLZ/GP and DME from the control tower via status panel. The panel should have the option of sounding an audible alarm in the event of failure and/or critical parameters being out of alignment. - An integrated monitoring system shall be provided that monitor and control the key parameter of all systems. The system shall allow for monitor and control from at least two remote locations. Key parameter includes but not limited to the following; - Remotely shut-down and restart - Remotely adjust critical parameters - Input power A/C and DC supply. - External back-up power source such as Batteries and battery chargers. - Environmental conditions (e.g. Room temperature) - The monitoring and control system shall store the history of all alarms and changes. - The monitoring and control system shall have at less two access levels. Level 2 for parameter changes and level 1 for read only access (monitoring). 5. Frangible Building - The tenderer shall demonstrate the frangibility of its system. For installations with overall height above 1.2 m the frangibility must be verified through a full-scale dynamic impact test or computer analysis supported by a representative field test. The support structure should not impose on the colliding aircraft a force in excess of 45 kN. 5.2 The Building Shall meet the ICAO Frangibility requirements outlined in Doc 9157. 6. Generators 6.1 The tenderer shall provide backup power facility at GP and LLZ. The generator capacity shall be 40% greater than the peak consumption the proposed equipment. 6.1 The tenderer shall ensure that the proposed generator has a local dealer in Jamaica. |  |

| SECTION D – SPARE PARTS |  |
| --- | --- |
| 1. SPARE PARTS 1.1 The supply of spares shall be based upon the manufacturer's proven reliability figures for the equipment type and shall reflect the reliability expected in the intended service and location. 1.2 Spares will consist of Level 2 spares (this is considered the LRU level) and other spares where repair can be performed without specialized test equipment. 1.3 The supply of spares shall be consistent with all the spare components, modules and sub-assemblies required to enable all the Level 2 maintenance actions required in order to ensure the rapid restoration of the supplied equipment, interfaces and systems to operational service. 1.4 The quality of spares shall be of the equal or exceed the quality of the parts included in the equipment originally installed, either from OEM or third party OEM certified. 1.5 The spares shall be supplied in their original packaging, duly protected against moisture with dehydrating agents or silica. Each set of spares shall be labeled with its part number and name and quantity of spares in each package. 1.6 The Tenderer shall include in its proposal a list of the most important modules, cards and components, indicating their name, manufacturer, part number, suggested quantity, unit price and total price, so as to ensure normal equipment operation during the five (5) years following the warranty expiration. 1.7 The Tenderer shall guarantee the availability of all spare parts and modules for the equipment under offer (both inclusive of and exclusive of those items in the recommended spares list) for a period of not less than Ten (10) years. 1.8 The Tenderer must indicate in its proposal the proposed procedures for supplying modules, parts and components on "quick-delivery" basis and procedures for repairing the PCBs and sub-assemblies. The estimated response time must be included. 1.9 Upon expiration of the warranty, the JCAA shall have the option of modifying the spare parts list to be acquired in accordance with the operational performance of the overall system. 2. ACCESSORIES AND CONSUMABLE GOODS 2.1 All accessories and cables necessary for the installation, integration, interfacing and maintenance of the equipment shall be provided. A detailed list of such accessories shall be provided as part of the Tenderer’s proposal. The JCAA reserves the right to purchase some, or all of the accessories detailed in such a list. 2.2 Where the above accessories are not included as standard items within the total cost of the system, but are available as options, each such item shall be identified and individually priced. 3 TOOLS AND TEST EQUIPMENT - The Tenderer shall include the supply of special tools, state of the art test equipment such as Oscilloscope, Network and logic Analyzers, Spectrum Analyzer, Power Meter, Nearfield Monitor, Portable ILS Receiver (PIR) and maintenance kits such as adapters, connectors, cables, PCB extenders, etc. required for installation, routine preventive and corrective maintenance and repair of all equipment supplied as part of this contract. 3.2 All the above equipment shall be delivered with two (2) copies of the relevant documentation necessary for its operation and maintenance. 3.3 An itemized and individually priced list of the proposed tools and test equipment is to be supplied as part of the tender documentation. 3.4 The JCAA reserves the right to purchase some or all of the items included in the list of proposed tools and test equipment. 4. DOCUMENTATION 4.1 The Tenderer shall provide three (3) sets of technical manuals in the English language, as well as one set on CD-ROM or Flash memory stick. 4.2 These manuals shall contain all information, at the module level, for the installation, operation and maintenance, parts and components list, of all equipment supplied under this project. All hardware and software that is furnished by third parties and integrated with the system shall have all of its support documentation delivered in the language of the country in which it was developed or manufactured. 4.3 The Tenderer shall include a set of technical manuals containing the description, replacements and main features of the system components. 4.4 The manuals shall include flow diagrams of the signals between the frames and the equipment, indicating the type and terminology used and the signal diagrams of any external interfaces. 4.5 The contractor shall also provide ten (10) sets of operational manuals in the English language covering the operational procedures of the systems, as well as two (2) soft copies in CD-ROM/Flash memory. 4.6 A CD/DVD in commercial format (Word, Excel, AUTOCAD, PDF, etc.) containing all the documentation in English including the following shall be supplied with the documentation: a) Installation and implementation drawings that may have been modified, if necessary, to the as built configuration. b) Electrical diagrams. c) All hardware documentation, the description of maintenance procedures, including the parts location drawings and hardware layout. d) All software documentation, including licenses. e) Maintenance, trouble shooting, training (operational and maintenance), testing and diagnostics materials. 4.7 The Tenderer shall provide complete information on the electrical interconnections at the equipment level, power supply systems, units and modules, as well as on the internal electrical energy system, lighting and air conditioning, specifying their location and distribution. 4.8 All manuals shall be current editions including any amendments applicable up to date of delivery. |  |

| SECTION E – SERVICES, TESTS AND ACCEPTANCE |  |
| --- | --- |
| ##### 1 Factory Acceptance Test 1.1 The Tenderer shall be required to test the system at his own premises prior to shipment. This Factory Acceptance Test shall be performed in accordance with the Tenderer’s standard factory acceptance test procedure. 1.2 The Tenderer shall give to the JCAA thirty (30) days prior written notice with the details of the time, date and place of factory acceptance tests. 1.3 The test procedures shall contain step-by-step instructions with an explanation of each test. 1.4 All test equipment used for the factory acceptance tests shall be standard commercial equipment, and all ancillary equipment required for testing shall be furnished by the tenderer for the duration of the tests. 1.5 Factory Acceptance shall be required for all equipment, before shipping. The Factory Acceptance shall verify all equipment functional and operational capabilities and physical characteristics as specified in the Scope of Compliance. 1.6 All results of the Factory Acceptance Test shall be duly recorded and shall be signed by the Tenderer’s representative and the JCAA. 1.7 All observations agreed on, and discrepancies noted during the Factory Acceptance Tests shall be corrected by the Tenderer before the SAT. 1.8 The tenderer shall arrange for one (1) FAT Session at the same location, to run consecutively for all equipment. All the sub-system FAT of one (1) selected system shall be carried out successively without interruption. 1.9 The JCAAs appointed representative(s) shall be entitled to enter the premises of the Tenderer at reasonable times during normal working hours to witness the test of the equipment and work in progress. 1.10 Notwithstanding any inspection or test conducted at the factory prior to shipment, all equipment shall be subject to Site Acceptance Tests. 1.11 The Factory Acceptance Tests shall be performed at the Tenderer’s factory in accordance with its standard procedures. Minor defects or shortcomings which can be corrected during the Installation shall not prevent the signature of the Factory Acceptance Certificate, to the extent the Tenderer undertakes to rectify such minor defect and/or shortcoming, as soon as practicable thereafter and at the latest before the Site Acceptance. 1.12 The Factory Acceptance Tests shall be conducted in the presence of the JCAA’s appointed representative(s) whose names shall be provided to the Tenderer at least two (2) weeks prior to the commencement of tests. Following the satisfactory completion of the FATs, the JCAA shall sign and issue a Factory Acceptance Certificate. 1.13 If the JCAA appointed representative does not issue and sign the Factory Acceptance Certificate (FAC), s/he shall immediately notify the Tenderer in writing with proper reference to any tests in the approved Acceptance Test schedule or to any part of the Specifications which the equipment has failed to meet. Minor failures, which do not adversely affect the performance or operation of the equipment for the purpose intended, and subsequently subject to modification by the Tenderer, shall not be considered as items preventing JCAA Factory Acceptance. 1.14 With respect to the JCAA’s reason for non-acceptance, the Tenderer shall give notice to the JCAA stating how it intends to rectify the faults identified in the equipment in order that the JCAA may repeat the tests with which the equipment did not initially comply with and to repeat also the tests in respect of those parts of the equipment affected by the rectification. 1.15 The equipment shall be considered factory accepted by JCAA upon satisfactory completion of each Acceptance Test as certified by the relevant test records signed by the Tenderer’s appointed representative and counter-signed by JCAA’s appointed representative(s). 1.16 This acceptance shall mean to all intents and purposes compliance on the part of the Tenderer with its contractual obligations and shall entitle it, after ninety (90) days following the Factory Acceptance certificate, to the release of the contract advance payment bond. 1.17 The Tenderer's proposal shall include the cost for airfare to and from JCAA (business class), travel medical insurance, hotel accommodation, ground transportation (hotel to factory) and terminal transportation (airport to hotel and return) and Daily Subsistence Allowance (DSA) based on International standard rate, by four (4) personnel from the JCAA. 1.18 The Tenderer shall deliver an operational factory orientation course for four (4) FAT attendees immediately prior the FAT. This course shall be sufficiently comprehensive to enable the FAT attendees to assess the performance of the system supplied. This training shall be focussed towards acquiring knowledge of the operation of the Display Site equipment and its associated equipment for which a detailed plan shall be presented with the tender, including the location where the orientation course shall take place, as well as the duration and the content. This training is separate from the Factory technical training. 2 Installation 2.1 The Tenderer shall ensure that there is no impact upon the existing Air Traffic Control service during installation, site testing and commissioning activities and for a determined period, agreed upon with JCAA, following the installation. 2.2 The Tenderer shall ensure that installation workmanship complies with the highest quality standard levels. 2.3 The Tenderer shall be responsible for the installation of all equipment, cabling and accessories. 2.4 All installation materials, services, personnel teams, test equipment and tools required for installation of the equipment shall be provided by the Tenderer. 2.5 The Tenderer shall individually identify all installation personnel in terms of qualifications, experience and designation. Individual names are not required at the tender stage. 2.6 Any spare parts used to replace faulty items during the installation and check out phases and until final acceptance of the equipment shall be replaced by the Tenderer free of charge. 3. Site Acceptance Test (SAT) and Flight Check - 3.1 After the Equipment has been installed and tuned, the Tenderer in the presence of the technical representative(s) appointed by JCAA, shall proceed with the Site Acceptance Tests. The Tenderer shall give the JCAA at least thirty (30) days prior written notice of the date on which the equipment shall be ready for the SAT. 3.2 The Tenderer shall demonstrate during SAT, using its standard procedures that the equipment provided within this project is compliant to the technical specification and requirements. The Tenderer shall provide the test equipment required for the SAT - 3.3 Within ten (10) days of the completion of SAT, where no defects, poor workmanship or malfunctions which prevent the equipment operating materially in accordance with the Technical Specification are found in the installed Equipment, the Site Acceptance Certificate (SAC) shall be signed by both parties. If the equipment is rejected by the JCAA, the JCAA shall issue within five (5) days of the completion of the Site Acceptance Test, a report to the Tenderer listing the shortcomings which led to the rejection. Upon such a rejection by the JCAA, the Tenderer shall make good on the shortcomings with all diligence and retest all equipment concerned for acceptance in conformity with the Site Acceptance Test. 3.4 This acceptance shall mean to all intents and purposes compliance on the part of the Tenderer with its contractual obligations. The two per cent (2 %) contract performance bond shall remain in effect until the end of the extended warranty period. - 3.5 JCAA shall provide an aircraft with flight calibration instruments and a qualified team, for flight testing, as soon as scheduling permits, after the successful completion of the SAT. JCAA shall provide an agreed upon number for hours of flight time for inspection. The JCAA and the Successful tenderer shall specify the fighting time requested. - 3.6 The Tenderer shall support the flight check exercise with on-site personnel. Should the flight check be deemed unacceptable by the JCAA or the Tenderer due to equipment malfunction, and a second flight check is required at a future date, the Tenderer shall correct the errors and redo the flight check at their own expenses associated with the second flight check. 4 On-the-Job Training (OJT) 4.1 OJT training of operational and maintenance personnel shall be provided by the Tenderer as part of the pre-commissioning activities for EIGHT (8) technical staff identified by the JCAA. 4.2 The OJT shall be conducted in English during the installation and /or testing phase of the project. 4.3 The Tenderer shall provide full details, including duration, schedule, system covered, etc. of the proposed OJT training package. 5 Delivery 5.1 The Tenderer shall be responsible for the necessary delivery and transportation arrangements for the Equipment (including Customs Clearance and nationalization including all associated costs with transportation to the installation site. 6 PROJECT MANAGEMENT 6.1 Preliminary Project Plan 6.1.1 The Tenderer shall submit with their proposal a preliminary project management plan and GANTT chart containing the design, manufacture, Factory Acceptance Tests (FAT), training, installation, Provisional Site Acceptance Tests (PSAT), Flight Checks, Operational Readiness Demonstration (ORD), and Final Site Acceptance Tests (FSAT) and commissioning tasks for review. 6.2 System Design Document 6.2.1 The successful Tenderer shall submit, within thirty (30) days of the signing of the contract by both parties, a System Design Document (SDD) which shall contain a detailed listing of the scope of supply and related costs, (including spare parts/accessories), technical/operational description of the proposed equipment, preliminary civil works, training plan, and the equipment functional and operational characteristics. The SDD shall be delivered in three (3) hard copy in English and also must also be provided on CD. This document shall be prepared using standard Microsoft tools (e.g. Word or Excel) or PDF. 6.2.2 The Successful Tender shall submit within thirty (30) days of the contract award the following items for the JCAA’s approval: - A complete and detailed final project work schedule. - A transition plan describing how the proposed equipment can be installed and commissioned with minimum disturbance and disruption to the existing air traffic services operations. - A calendar presented in weekly segments from contract award to commissioning with the following major milestones: - Preliminary design review, - Final design review, - Factory manufacturing/assembly, - FAT, - Civil works, - Site preparation, - Training, - Installation, - Provisional SAT, - ORD, - Final SAT, - Flight check, and - Commissioning. 7.3 The System Design Document and program schedule shall be subject to the JCAA’s prior written approval. If the document(s) in question are rejected by the JCAA, the Tenderer shall have ten (10) business days to correct the document(s) and revise them at no additional cost to the JCAA. 7.4 Sixty (60) days after signing of the contract by both parties, the Tenderer shall submit the necessary and required studies, water, sanitary and structural drawings, and the definitive architectural drawings of all proposed civil works to the JCAA for review and approval. 7.5 The Tenderer shall, at no extra cost, update the approved project plan on a monthly basis until contract completion. 8. SCHEDULE OF IMPLEMENTATION 8.1 The Tenderer shall ensure that the overall project implementation and commissioning does not exceed twelve (12) months after contract signing. |  |